Synthesis and characterization of polyacrylonitrile and carbon nanotube composite with homogeneously dispersed carbon nanotubes

Muia Lilian Mutheu

student, Nanotechnology and microsystem technics

National university of science and technology MISiS, college of new materials and nanotechnology, Moscow, Russia

Email: lmuia89@gmail.com

Carbon nanotubes have been known to improve the properties of polymers such as tensile strength, modulus, electrical conductivity, chemical resistance and thermal stability of polymers [1]. Of these polymers, polyacrylonitrile (PAN) has been widely used as a precursor due to its high carbon yield [2] compared to other precursors such as pitch and rayon [3, 4] however the main challenge up to date has been to uniformly disperse the carbon nanotubes (CNTs) in the polymer matrix [5].

In this study it is shown that a solution of PAN with three absorbance bands in the ultraviolet spectrum in dimethylformamide (DMF) and single walled carbon nanotubes (SWCNTs) can be used to form a composite with uniformly dispersed nanotubes using ultrasound treatment. The sonicated homogenous solution of PAN and SWCNTs in DMF was used to form a film which under infrared spectroscopy showed chemical bonding between the polymer and carbon nanotubes.

This composite could find application as a three dimensional printing material, in energy storage, electrostatic painting, field effect transistors, aerospace, and photovoltaics and electromagnetic interference shielding [5, 6].

References

- Sookhyun Hwang, Sangwoo Kim, Gyu-bong Cho, Hyonkwang Choi, Carbon nanotubes radially anchored on carbon fibers formed by polyacrylonitrile, Materials Research Bulletin, Volume 97, 2018, Pages 49-55

- Nataraj S.K., Yang K.S., Aminabhavi T.M., Polyacrylonitrile-based nanofibers—A state-of-the-art review, In Progress in Polymer Science, Volume 37, Issue 3, 2012, Pages 487-513

- L. Laffont, M. Monthioux, V. Serin, R.B. Mathur, C. Guimon, M.F. Guimon, An EELS study of the structural and chemical transformation of PAN polymer to solid carbon, Carbon, Volume 42, Issues 12–13, 2004, Pages 2485-2494

- Ok-Kyung Park, Sungho Lee, Han-Ik Joh, Jun Kyung Kim, Phil-Hyun Kang, Joog Hee Lee, Bon-Cheol Ku, Effect of functional groups of carbon nanotubes on the cyclization mechanism of polyacrylonitrile (PAN), Polymer, Volume 53, Issue 11, 2012, Pages 2168-2174

- Zdenko Spitalsky, Dimitrios Tasis, Konstantinos Papagelis, Costas Galiotis, Carbon nanotube–polymer composites: Chemistry, processing, mechanical and electrical properties, Progress in Polymer Science, Volume 35, Issue 3, 2010, Pages 357-401

- Arvind H. Jadhav, Xuan Thang Mai, Frank A. Ofori, Hern Kim, Preparation, characterization, and kinetic study of end opened carbon nanotubes incorporated polyacrylonitrile electrospun nanofibers for the adsorption of pyrene from aqueous solution, Chemical Engineering Journal, Volume 259, 2015, Pages 348-356