Optimal error-tolerant design of programmable multiport

interferometers

Fldzhyan S.A., Saygin M.Yu. , Kulik S.P.

student,

Faculty of Physics, M.V. Lomonosov Moscow State University, GSP-1, Leninskie gory, Moscow 119991 Russian Federation
E–mail: apres.suren@gmail.com

Photonic information processing often demands programmable multiport interferom-

eters capable of implementing arbitrary transfer matrices, for which planar meshes of

tunable Mach-Zehnder interferometers (MZIs) are usually exploited [1], [2]. However,

these MZI-based interferometers require balanced static beam-splitter (BSs) that make

them sensitive to manufacturing errors. Here, we abandon the error-sensitive MZI and

propose an alternative design that uses a single static BS and a variable phase shift as the

building block of the interferometer mesh. Our BS-based design [3] (wich strongly utilizes

the ideas in [4]) has been shown to possess superior resilience to manufacturing errors,

which is achieved without addition of extra elements into the schemes. Namely, the power

transmissivities of the static BSs constituent the interferometers can take arbitrary values

— phase shift

— beam-splitter

in the range from ≈ 1/2 to ≈ 4/5.

| Figure 1: The novel design scheme for N = 5 multiport interferometer. The basic 2-port interferometer transformation constists of invariable BS and configurable phase shift. |  |
| --- | --- |

This work was supported by a grant of The Russian Federation Ministry of Educa-

tion and Science.

References

- M. Reck, A. Zeilinger, H. J. Bernstein, and P. Bertani// Physical Review Letters 73, 58 (1994).

- W. R. Clements, P. C. Humphreys, B. J. Metcalf, W. S. Kolthammer, and I. A. Walsmley// Optica 3, 1460 (2016).

- S. Fldzhyan, M. Saygin, S. Kulik// arXiv e-prints, arXiv:1910.13001 (2019)

- M. Saygin, I. Kondratyev, I. Dyakonov, S. Mironov, S. Straupe, and S. Kulik// (2020) Physical Review Letters. 124.10.1103/PhysRevLett.124.010501.